S m a l l  -   C     D o c u m e n t a t i o n

V e r s i o n     2.70     10/30/86

by

F. A. Scacchitti

25 Glenview Lane

Rochester, NY 14609

(716) 482 - 7159

This  document  is intended to supplement J.  E.  Hendrix's  "The

Small  C Handbook" and will only discuss the differences  between

this version of Small C and that described in the Manual.  It  is

recommended  the  user  purchase  Hendrix's  manual  for  a  more

complete treatment of Small C.

For information regarding the contents and structure of the Small

C library, refer to the file CLIB.DOC and Hendrix's Manual.

This  compiler was originally Version 2.1 as obtained from  J.  E

Hendrix.  It  includes all of his newsletter  fixes/enhancements.

Refer  to the file CC.DEF for a complete chronological history of

changes and details regarding modules or functions changed.  Also

it  is  assumed the user is somewhat familiar  with  the  M80/L80

software.  The  compiler  produces  an output  file  of  assembly

language pneumonics which must be assembled and linked.

This  compiler  is  a superset of J.  E.  Hendrix's  Version  2.1

compiler,  yet  still a subset of C as described by Kernigan  and

Ritchie in "The C Programming Manual". The following enhancements

have been installed:

2.1 --> 2.2

* - CP/M runtime package and I/O library to optimize  size

and speed

* - Global initialization option -i

* - No boot option -n

* - external static Type Specifiers

* - Conditional operator {expr1 ? expr2 : expr3}

2.2 --> 2.3

* - added multiple levels of indirection

eg. **char c; ***int ident;  ****var = **c;

2.3 --> 2.4

* - improved multiple levels of indirection to ensure

appropriate element is stored or retrieved

2.4 --> 2.5

* - added arrays of pointers

eg. char *argv[]; int **i[20]; *i[2] = 22;

* - added hex and octal constants

eg. 0x1a, 0XFA, 03777, 00017

2.5 --> 2.6

* - added global multidimensional arrays

eg. char arc[5][5]; int i[3][4][5]; var=i[2][0][3];

2.6 --> 2.7 (Released 10/86 for public consumption)

* - added nested includes

* - new runtime library (rdrtl.rel) for compiler and/or

redirected output

* - new i/o library (clib.rel)

Planned   2.7 --> 2.8 (In final stages)

* - install floating point (float and double)

(only for Z80 microprocessors)

* - allow function definition syntax

* - add compiler error tracker/counter

Planned   2.8 --> 2.9 (schemes developed)

* - add typedefs, sizeof, unsigned integers

Planned   2.9 --> 3.0 (matter of parsing) (2nd quarter '87)

* - allow auto, register, short declarations

(This will be a release version and the final one

for CP/M)

Planned   CP/M 3.0 --> MSDOS 3.0 (3rd quarter '87)

* - using J. E. MSDOS version upgrade to my version 3.0

(will contain floating point, if I can obtain a good

package for MSDOS)

Planned   MSDOS 3.0 --> MSDOS 4.0 (not started) (4th quarter '87)

* - add longs, unions and structures

This  is  an  aggressive schedule (slips are  possible),  but  at

completion  should be a full implementation of C.  If anyone  has

any  suggestions or has already accomplished that  which  remains

undone here, don't hesitate to contact me. I have no qualms about

collaborating to finish the job or optimize compiler operation.

FEATURES and ENHANCEMENTS

* - Global initialization option -i

This  feature decreases compilation time,  especially when  large

arrays are declared globally.  Most versions of the compiler  (at

least  the  ones I've seen),  by default,  initialize all  global

variables to zero by generating either DB 0 or DW 0.  Arrays  are

initialized via DB 0, 0, 0, 0, 0, 0, . . . . . . . . . .

This  adds significantly to compile time if the user has declared

large  global  arrays.  The  compiler  now issues  a  DS  n  thus

allocating n bytes of uninitialized memory.  If the -i switch  is

used,  memory  will  be  initialized to zeroes as  before.  As  a

furthur  example, the Eratosthenes Prime Number  Sieve  takes  80

seconds to compile when the -i switch is used but only 17 seconds

if  it isn't.  An additional 55 seconds is required with each for

assembly and linking.

* - No boot option -n

This  feature allows rapid return to the CP/M prompt  on  program

completion  at a cost of 800h bytes of code space.  It is used in

conjunction with the runtime package in the C library (CLIB.REL).

If  the the -n switch is used and the program contains  a  main()

function,  DB ZZZCCP 1 will be generated by the compiler prior to

the  END statement.  If it isn't specified the compiler generates

DB  ZZZCCP  0.  The global variable ZZZCCP determines  where  the

stack should be placed and the return path to CP/M.  In short, if

the -n option is used the stack is placed at the base of the  CCP

and the program returns to CP/M via a return instruction.  If the

-n  option isn't specified the stack is placed at the base of the

BDOS and return is performed via a warm boot.  Refer to  CLIB.DOC

and  ULINK.MAC  for details regarding operation of this  feature.

This feature is NOT recommended for use with programs  performing

disk i/o, but was intended primarily for non-disk utilities.

* - external static Type Specifiers

Global  variables  are more rapidly accessed (both  fetching  and

storing)  and require less code to access than  local  variables.

Refer   to  Chapter  19  of  Hendrix's  manual  for  a   complete

explanation.  Most Small C compilers make all global declarations

public.  This  obviously  presents a problem (especially for  the

variables c and i) in taking full advantage of this feature.  The

static type specifier implies that the variable should be  memory

resident, yet known only to the module it is compiled in. This is

accomplished  simply  by  generating  a single  colon  for  types

specified as static and the original double colon for those  that

are  not.  This  results in variables that are local only to  the

file  in which they are compiled.  It is ideal for  non-recursive

library functions.

* - Conditional operator {expr1 ? expr2 : expr3}

This  is just another enhancement to bring Small C closer to  Big

C.  This  operator  is described quite thoroughly in  Kernigan  &

Ritchie's  "The  C Programming Language" and works as  specified.

The  user  should  try the examples in  thier  manual.  The  code

generated  by this operator is almost exactly the same as an  if-

else statement, however if does lead to more succinct and elegant

code  at the source level and can be used inside statements where

if - else fears to tread.

* - added multiple levels of indirection

This was always a sore spot with me.  Now your program can use  a

standard  argc,  argv syntax.  (int argc;  char **argv;) It  also

generates  more  efficient  and  shorter  code  since  additional

argument  assignments  aren't necessary when  accessing  extended

levels of a pointer.

* - added arrays of pointers

One more step toward standardization with full C.  The programmer

may use char **argv;  or char *argv[];  and/or retrieve/store  as

specified by K & R.

* - added hex and octal constants

Works as described by K & R. NO longer any need for that decimal-

hexadecimal-octal calculator on the shelf. (Well at least not for

figuring contants and addresses for your small c program.)

* - added global multidimensional arrays

Some  programs almost demand this capability.  However,  there is

some overhead involved in using multi-dimmed arrays and the  code

generated  can  be  larger than if  multiple  single  dimensioned

arrays are used. The generated code calculates the offset for the

particular  element  be  accessed.  The user must decide  if  the

overhead is worth allowing the compiler to do the book keeping.

* - added nested includes

Simple enough to add as long as the file structure permits.  This

particlar implementation allows nesting six deep.

* - new runtime library (rdrtl.rel)

The Runtime Module used with this compiler was originally written

in C and contained assembly language enhancements.(I think !) The

present  module  RDRTL.MAC  contains  most of  the  i/o  routines

required by the compiler and is structured around the 8 - 1k file

buffers. Ouput redirection (via > and >>), argv/argc handling and

special  abort features are included.  The resultant size of  the

compiler  after linking  CCRTL.REL,  CC1.REL,  CC2.REL,  CC3.REL,

CC4.REL,  and  searching  CLIB.REL  to pick up  a  few  necessary

function is < 31K.  Smaller than most (Version 2.7).  This module

can   be  used  with  the  compiler  or  any  program   requiring

redirectable  output.  It should must always be linked in  first,

before  searching the clib module.  Failing to do so will provide

unpredictable results.

* - new i/o library (clib.rel)

Installed { exp1 ?  exp2 :  exp3 } operator and external  statics

where appropriate. Library size was reduced from 26k to 24k. Also

fixed all known bugs and corrected non-standard operations.

** - installing floating point (float and double)

In the works.  I'm working with the same package installed by Dr.

James  VanZandt and using his Small-C Version 1.2  implementation

as  a guideline.  It requires a Z80 processor or another floating

point package.  The code generated is generic and could work with

many float packages, the one I'm using is a 6 bit implementation.

** - allow function definition syntax

Syntax  such  as  "FILE *fd *fopen();" during  global  and  local

declaration  or  "int func(var1)char var1;" to define a  function

will  cause erroneous multi-define errors.  This problem will  be

cleared up and said syntax is now allowed.

** - add compiler error tracker/counter

At  completion of compiling the compiler will reports the  number

of  errors that occurred.  This could be extended to evalute  the

results  and determine whether processing  should  continue.  ie.

assembly, linking, optimization . . .

** - add typedefs, sizeof, unsigned integers

typedefs  are  a little tricky but ca be accomplished by using  a

convoluted version of the #define code. Don't worry they are true

type definitions.

sizeof will be almost a duplicate copy of the method employed  by

J. E. Hendrix in his MSDOS Version 2.1.

unsigned  integers require adding a byte to the symbol table  for

furthur variable definition. Most of the routines were already in

place, just not employed.

** - allow auto, register, short declarations

This  was just a matter of adding the appropriate syntax to allow

parsing these keywords and verifying proper usage.  Both auto and

register must be locals while short must define an integer.

*  fully implemented

**  At some interim stage, alpha test, beta test, debug, or under

development.

USING THE COMPILER

The  major differences between using this compiler and  Hendrix's

are  (1) this compiler does not create a default output file  and

(2)  his  runtime package includes redirectable input and  output

(and  the  resultant  code size overhead). If no  output file  is

specified,  the  output  will go to the console where it  may  be

stopped and started via Control S and Control Q. If the output is

suspended  by  Control  S,  Control C may be used  to  abort  the

compile.  The  compiler is invoked by running the program  cc.com

followed by the desired options (in the runstring). Some examples

follow:

cc

compile console input to console output

cc test.c

compile test.c direct output to console

cc test.c -p -a -l2

compile test.c direct output and source to console

pause on errors and sound bell

cc test.c >test.mac

compile test.c direct output to file test.mac

cc test.c >test.mac -p -a

compile test.c direct output to the file test.mac

pause on errors and sound bell

cc test.c new.c >test.mac -l2 -i

compile test.c and new.c direct output to test.mac

display source on screen

initialize global variables to zero

cc test.c >test.mac -o -n -l1

compile test.c direct output to test.mac

optimize for size over speed

generate no boot flag

insert source in output file as comments

cc test.c >test.mac -a -p -l2

compile test.c direct output to test.mac

display source on screen

pause on errors and sound bell

cc test.c >test.mac -o -n -l1 -p -a -i

compile test.c direct output to test.mac

optimize for size over speed

generate no boot flag

insert source in output file as comments

pause on errors and sound bell

initialize global variables to zero

COMPILER OPTIONS (SWITCHES)

-a   sound alarm(bell) on errors

-i   initialize global variables to zero

-l1  insert as comments in output file *

-l2  display source on screen as compile progresses *

-n   set noboot flag

-o   optimize for size over speed

-p   pause on errors

* only one of these may be specified

COMPILING THE COMPILER

Compiling the compiler is no different then compiling any other c

program.  What is different than normal is the linking procedure.

The file RDRTL.REL (ReDirectable Run Time Library) must be linked

first  to ensure that module satisfies its entry points prior  to

searching CLIB.REL.

The submit file CCCC.SUB provides an example of proper compiling,

assembling and linking of of this compiler. It's the one I use.

SETTING UP A COMPILER DISK

Four submit files are included with the source,  CCM.SUB, CCR.SUB

CCC.SUB  and  RCC.SUB.   These  files  will  produce  m80  source

files(.mac), l80 source files(.rel) and a final object file(.com)

without or with redirectable output,  respectively.  The user  is

encouraged  to  examine and/or modify these files to meet his  or

her own particular needs.

1	Obtain a print out of this file (CC.DOC)

2 	Initialize two disks.  One at the same capacity of  this

disk  (the  working disk) and the other  at  the  maximum

capacity of your system (the C disk).

3 	Copy the original disk to the working disk and store the

original away for safe keeping.

4       Sysgen your C disk, then install PIP (or SWEEP), M80, L80,

STAT, SUBMIT, and your current Editor.

5       Place this disk in Drive A and Warm Boot then PIP or SWEEP

the follow files from the working disk to the C disk.

STDIO.H          Header File

CC.COM	          Small C Compiler

CC.DOC           This file

CLIB.REL         C I/O Library

RDRTL.REL        Redirectable Output Library

CCM.SUB          Submit file to produce .MAC file

CCR.SUB          Submit file to produce .REL file

CCC.SUB          Submit file to produce .COM file

RCC.SUB          Submit file to produce .COM file

but links in the redirect lib first

You now have a working C Compiler Disk. Type in and enter

<STAT *.* $R/O> to prevent inadvertent erasures.

6       PIP or SWEEP one or more of the C source files to the

C disk.

FA.C       File Append Prog.

FB.C       File Copy Prog. (Binary or Ascii)

FC.C       File Copy Prog. (Ascii)

FM.C       File Mod Prog.

CAT.C      File Concatenate Prog.

FFD.C      Formfeed Prog.

HELLO.C    You gotta know this one.

ARGS.C     Example Argument Passing Prog.

ERATOS.C   Eratos Prime Number Sieve Prog.

TIME.C     Xerox 820 Time Set/Display Prog.

UW.C       File Un-Wordstar Prog.

WC.C       Word Count Prog. (use -i switch)

7       Type in and enter <SUBMIT CCC ERATOS> (or any source file)

The Eratos Benchmark will compile before your very eyes.

After the CP/M prompt returns, Type in and enter "ERATOS".

Neat huh ?

8       Have at it. Review the doc files supplied as well as the

manual written by J. E. Hendrix.

9       This Version of the C Compiler is offered free of charge

for non commercial use. However, donations are accepted

and will be divided to compensate this author as well as

the original to furthur the development of this compiler.

If there are any questions or problems regarding the use

of this version, please contact me. Address comments,

criticism, or suggestions to

Fred A. Scacchitti

25 Glenview Lane

Rochester, NY

14609

#################################################################################################################################################################################