FILENAME: DALLAS.DOC   UPDATED 14-July-92 by Don McKenzie.

Dallas Semiconductor products are available from:

VELTEK PTY/LTD

VIC (03) 808-7511       18 HARKER ST BURWOOD 3125  FAX (03) 808-5473

NSW (02) 713-4100       SUITE 12   50 GREAT NORTH RD. FIVE DOCK 2045

SA  (08) 232-3922       1ST FLOOR 176 GILLES ST ADELAIDE SA 5000

WA  (09) 351-9420       UNIT 5 64-66 KENT ST CANNINGTON WA 6107

_______________________________________________________________

14-JULY-92      Just popped into VELTEK BURWOOD and picked up the

1992-93 Edition of the Dallas Semiconductor Product Data Book.

D.S. have a data book with nearly 1000 pages that set me back $12. It

would have to be the greatest $12 worth of value I have seen for a long

time. I have been making this one of my "Bible" books for many years

now.

MAXIM semi-conductors is another must have book. The new Edition

available in about 6 weeks from VELTEK....

If you are into micro controllers or any computer related hardware

development, then you MUST have the DALLAS data book. It is packed with

a large range of unbelievable products that will save you time and

money. Top marks to the D.S. engineers.

_______________________________________________________________

SOME D.S. PRODUCTS....

The DS1213C is called a smart socket. It is a 28 pin device, or perhaps I

should be calling it a 56 pin device. It has legs below like any I.C. but it

has a socket on top. This allows you to plug it into a 28 pin socket, and

plug a 28 pin device into it. It takes up about the same board area as a 28

pin socket and allows you to space I.C.s at now standard .1" centres apart.

It increases normal chip height by approx. .37" so this dimension must be

allowed for in overall component height if this is a factor in your design.

All pins pass through the device except pins 20 and 28. Pin 20 is a

conditioned chip enable and pin 28 is a switched Vcc. The smart socket

monitors incoming Vcc for an out-of-tolerance condition. When this occurs,

an internal lithium source is automatically switched on and write protect is

enabled to prevent garbled data. Low power CMOS SRAMS should allow the data

retention time to be greater than 10 years. Both 6264 (8K) and 62256 (256K)

SRAMS can be used with this device.

The DS1225Y is 64K Non-volatile RAM. this device is almost equivalent to a

standard low power 6264 and the DS1213C, all in one package. It is approx.

.4" high. The only difference is pin 26 which had me going for a while. On

the DS 1225Y, pin 26 is not connected. A 6264 has pin 26 as an additional

high chip select. In most cases it shouldn't matter, but my PBUFF board

demands that pin 26 be a high select. This is part of my address decoding

logic, so this device can't be used in PBUFF without more decoding

circuitry. The DS1213C can be installed into PBUFF if required. Both

variations can be used in the ZLOAD board, as the address decoding logic is

on board.

The DS1216C is a smartwatch that is functionally equivalent to the DS1213C

but it has additional features. It keeps track of hundreths of seconds,

seconds, minutes, hours, days, date of the month, correct number of days in

month, months, years, am-pm mode, 12-24 hour mode, and caters for leap

years. Now that's what I call a watch. The setting and reading of the watch

is transparent to the SRAM. This device will plug into the PBUFF SRAM socket

without modification as pin 26 is straight through, the same as the DS1213C.

At the time of writing, I haven't written any software for the smartwatch,

but it is one job that I am looking forward to doing, even if I just get it

to log to the printer every 10 seconds as a demo package. I'm sure I'll

eventually think of a very useful project for it. I bet your mind is ticking

too!!

OTHER PRODUCTS....

Non-volatile sram.  16K, 64K, 256K, 1024K

Smartsockets.       16/64K, 64/256K, 64/256/1M

Smartwatch-rams.    16/64K, 64/256K, 64/256/1M

Smartwatch-roms.    256K, 1M

AND MORE....

Silicon timed circuits, Microcontrollers, Numerous time keepers and watch

dog timers, User insertable memory, Electronic tag, Key ring, PC port,

Security products, Integrated battery backup managers, System expansion,

Dram and SRAM sipstik products, Wireless products, Telecommunications, and

Modems.

With nearly 1000 pages to read, I haven't yet had the opportunity to

find out what all these things do, but if they are anything like the

smart products, I'm sure they will be very interesting.